TX531

CPU Auto-Voltage Detection Design

User’s Manual

451-053100000

REVISION: D

01-Oct-1997

Printed in Taiwan

Declaration  of  Conformity

Application of Council Directive(s):

89/336/EEC

Standard  to which Conformity is Declared:

en 55022:1994/a1:1995 Class B

en 50082-1 (iec 801-2:1984, iec 801-3:1984, iec 801-4:1988)

Manufacturer :	DataExpert Corporation

Address :	Breguetlaan 1, 1438 BA Oude Meer, The Netherlands

Telephone : 	+31 (20)-653-2243      Fax: +31 (20)-653-2265

Type of Product : 	Motherboard

Model No. :	TX531

I, the undersigned, hereby declare that the equipment specified above conforms to the above Directive and Standards.

S I G N A T U R E

V ice   P r e s i d e n t

P O S I T I O N     T I T L E

July, 4, 1997

d a t e   o f    I s s u e

User’s  Notice

No part of this product, including the hardware and software may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language in any form by any means without the express written permission of  DataExpert Corporation. (hereinafter referred to as DataExpert) except documentation kept by the purchaser for backup purposed.

Specifications are subject to changes without notice,  DataExpert provides this manual “as is” without warranty of any kind, either express or implied, including but  not limited to the implied warranties or conditions of merchantability or fitness for a particular purpose.  In no event shall DataExpert be liable for any loss or profits, loss of business, loss of use or data, interruption of business, or for indirect, special , incidental, or consequential damages of any kind, even if DataExpert has been advised of the possibility of such damages arising form any defect or error in this manual or product.  DataExpert may revise this manual from time to time without notice.

Products mentioned in this manual are mentioned for identification purposes only.  Product names appearing in this manual may or may not be registered trademarks or copyrights of their respective companies.

- Intel and Pentium are registered trademarks of Intel Corporation.

- Windows and MS-DOS are registered trademarks of Microsoft Corporation.

The product name and revision number are both printed on the board itself.  Manual revisions are released for each board design represented by the digit before and after the period of the manual revision number.  Manual updates are represented by the revision letter shown on the back of this manual.  For previous or updated BIOS, drivers, or product release information you may visit our home page at: http://www.dataexpert.com.tw/.

Installation  Guide

CPU - It is recommended for installation of AMD K6 MMX 233 and Cyrix M1 PR200 CPU to be installed with certified CPU Cooler (Big Cooler) from the approved suppliers.  Insufficient Cooling for the CPU may render the system unstable.  Our BIOS will check for the ID code of the CPU and if the CPU ID code do not match your CPU marking, then return them as it is re-marked from the original marking.  Check that the CPU is firmly installed on the CPU socket and CPU Cooler secured before powering up the system.

Memory - Best Recommended for 60ns EDO DRAM on SIMMs memory.  Do not mixed SDRAM (3.3V) with EDO 5V type of memory.  Please check for re-marked 60ns EDO DRAM by visual inspection of the surface on the components.  Most re-marked memory will shows polished surface.  We do not guarantee reliability for re-marked memory.  For BIOS setting for DRAM configuration, it is advisable to set the default setting for initial checking of the system.  Good Performance needs all installed configuration device installed to be of excellent quality including memory, hard disk, VGA and others.

Hard Disk - For best Performance, it is best recommended to install Ultra DMA/33 mode hard disk.  It works in conjunction with Ultra DMA 33 drivers on our utility disk attached.

VGA - For best compatibility, TX531 has been tested with famous VGA brand like ExpertColor VGA, Diamond, ATI, Matrox series.  The VGA should be 4 layers PCB type for good display reliability with certified EMI test like CE or DOC (FCC).

BIOS - Setup the system as “Optimal default Setting”.

Cables - The TX531 has prevention of reverse connection with connector bracket.    Check each of the cable has been connected to their respective devices before power-up.

Trademarks and/or registered trademarks are the properties of their respective owners:

The information in this publication has been carefully for reliability; however, no responsibility is assumed for inaccuracies.  Specifications are subject to change without notice.

IBM, PC/AT, and PC/XT are trademarks of International Business Machines Corporation.

Pentium is a trademark of Intel Corporation

MS-DOS and WINDOWS NT are register trademarks of Microsoft Corporation.

Cyrix is a trademark of Cyrix Corporation

AMD is a trademark of Advanced Micro Devices, Inc.

Static Electricity Precautions

Static electricity can easily damage your mainboard.  Observing a few basic precautions can help you safeguard against damage that could result in expensive repairs.  Follow the measures below to protect your equipment from static discharge:

- Keep the mainboard and other system computers in their antistatic packaging until you are ready to install them.

- Touch a grounded surface before you remove any system component from its protective anti-static packaging.  A grounded surface within easy reach is the expansion slot covers at the rear of the system case.  Or any other unpainted portion of the system chassis.

- During configuration and installation, touch a grounded surface frequently to discharge any static electric charge that may build up in your body.  Another option is to wear a grounding wrist strap.

- When handing a mainboard or an adapter card, avoid touching its components.  Handle the mainboard and adapter cards either by the edges or by the mounting bracket that attaches to the slot opening in the case.

Unpacking the Mainboard

The mainboard comes packed in a sturdy cardboard shipping carton.  The carton contains:

- The Mainboard

- This User’s Manual

Note: Do not remove the mainboard from its original packing until you are ready 	to install it.

The mainboard is easily damaged by static electricity.  Observe the following precautions while unpacking and installing the mainboard.

- Touch an unpainted area of system chassis before handling the mainboard or any component.  Doing so discharge the static charge your body may have built.

- Remove the mainboard for appearance checking.  Shipping may have lossened integrated circuits from their sockets.  If any integrated circuit appears loose, press carefully to seat it firmly in its socket.

Do not apply power if the mainboard appears damaged.  If there is damage to the board, or items are missing, contact your dealer immediately.

Power Precautions

Before you begin configuration, make sure you are working with an unplugged mainboard.  Many components are powered by low-voltage current, but there still may be a dangerous electric current coming from the leads and power supply.  You should take the following precautions:

- Turn off the power supply, and unplug the power cord before you begin.

- Unplug all canles that connect the mainboard to any external devices.

Note: Before making connections to the board, make sure that power to the system is turned off.

Table of Contents

Chapter  1   Introduction	1

1.1	Overview	1

1.2	System Features	2

1.3	System Specification	3

1.4	System Performance	5

Chapter 2    Installation	6

2.1	Installing DRAM	6

2.2	TX531 Board Layout	9

2.3	Setting Other Jumpers & Connectors	10

2.4	Configuring CPU	14

2.5	Installing Flash ROM	19

2.6	Installing Flash ROM Utility	20

Chapter  3   System BIOS Setup	21

3.1	Standard CMOS Setup	22

3.2	Advanced CMOS Setup	24

3.3	Advanced Chipset Seup	29

3.4	Power Management Setup	32

3.5	PCI/PnP Setup	34

3.6	Peripheral Setup	35

Chapter  1    Introduction

1.1    Overview

The TX531 motherboard optimized for MMX CPUs for INTEL, AMD K6-Write Allocation feature and Cyrix M2-Linear Wrap Mode incorporates the Auto-Voltage detection and DIP switches for CPU fast and easy installation.  With on-board 512K L2-write-back cache, it provides workstation level computing performance at PC system level.

The TX531 provides highly efficient bus transaction, highly concurrent architecture and highly integrated design with MESI tag memory built-in, USB, PS/2 KBC, 2 channel dedicated Ultra DMA/33 IDE Master, with the best power management and ACPI.  Also built-in is Super I/O that supports Win95 plug-and-play function.

The TX531 also supports all Pentium class CPU family and supports host frequency up to 75MHz and 83.3MHz.  Its design provides Auto-banking for memory expansion with 4x SIMMs or 2x DIMMs sockets up to 256MB of DRAM.

The TX531 motherboard provides outstanding I/O capabilities with 4x PCI 2.1 bus mastering slots for a high bandwidth data path for data-movement intensive functions such as graphics acceleration and all its advanced features for a cost effective, high performance, and highly expandable platform which delivers the latest CPU and I/O technologies.

1.2    System Features

The TX531 has many performance and system features integrated into the motherboard, including the following:

- AT Form-Factor ACER PC’97 compliance (Intel 430TX compatible) chipset Pentium-based Motherboard with 512K Pipeline Burst SRAM Cache.

- Supports Intel Pentium Family CPU running at 90MHz~200MHz (P54C), Intel Pentium with MMX 166/200/233MHz (P55C), AMD K5 Pentium-level CPU 100/133/166, AMD K6 166/200/233 MMX CPU and Cyrix 6x86 133/150/166/200 & 6x86MX.

- Integrated Programmable 2 fast serials (16550) / 1 Parallel (Standard/ECP/EPP) / FDC controller / PS/2 Mouse Port, 2 x PCI Bus Master (Ultra DMA/33 Mode) IDE controllers.

- Provides 3 x 16-bit ISA slot & 4 x Concurrent PCI 2.1 expansion slot.

- Supports Auto-banking 8MB~256MB Memory utilizing 4 x 72 pin EDO/FPM SIMMs or 2 x 3.3V or 5V 168-pin DIMMs for SDRAM expansion.

- Support Flash BIOS for Plug & Play/DMI/Anti-virus/LS120 technology (120MB Capacity) features to provide an ease-of-use add-on cards configuration for users.

- Supports optional Universal Serial Bus Port and Infrared IR port (115.2 Kbps/Sec) pin header ready.

- Provides CPU Auto Voltage detection for fast and easy CPU installation and upgrade. Low Thermal switching voltage design for reliability.

1.3    System Specification

Multi-Processor Support:  Intel Pentium 90~200 MHz, Intel Pentium with MMX 166~233 MHz

AMD-K5 (90-166 MHz), AMD-K6 (166~233 MHz) Cyrix 6x86 (120+~200+ MHz) & Cyrix 6x86MX

CPU Clock Speed:	50/55/60/66/75 MHz

DRAM:	8MB to 256MB (Auto Banking)

SRAM:	512K L2 Cache

BIOS type:	AMI  BIOS with built-in CPU Auto-voltage detection  	capabilities

Slot types:	Three 16-bit ISA slots + four 32-bit PCI 2.1 slots

Dimension:	22x25 cm (AT form factor)

Utilities Disk:	DMI for LAN management, Ultra DMA/33 Drivers, Acer Chipset ID setup

Additional Features

Miscellaneous Button: 	Reset button, Turbo LED, HDD LED

Board Design: 	4-layer Implementation for low noise operation

Utility Drivers

The utility disk contains drivers for the TX531 motherboard, including DOS, Windows 3.x, Windows 95, Windows NT3.5x, NT4.0, Novell Netware, and IBM OS/2.

To install DOS/Windows 3.x drivers, just run the batch file named “install.bat”.  To install drivers for other OSes, please refer to the readme file in certain directory.

Windows 95 Drivers Installation

- Popup ‘Control Panel’ and open ‘Add New Hardware’.

- Click on “Next”.

- Click “No” - don’t let Windows to search for your new hardware, then click “Next”.

- Select “Hard disk controller” and click “Next”.

- Click the button “Have Disk” (or press Alt+H).

- Make sure the install disk is in drive A (or drive B), and then click “OK”.

- The “Select Device” dialog box will appear.

- Select the device that you wish to install and click “Next”.

- After this install procedure is complete, you may need to restart the computer.

Attention:

- If you have any DOS Real-mode driver on your system.  It will be conflict between ACER Bus Master IDE Driver and a Real-mode ATAPI CD-ROM or IDE device driver.  ALL REFERENCES TO REAL-MODE DRIVERS (IDE or ATAPI) in the AUTOEXEC.BAT and CONFIG.SYS should be removed.

- Sometimes, the ACER IDE controller has already been installed with the standard driver provided by Microsoft.  In order to use ACER IDE controller with the driver we provided, it is necessary to remove the standard driver from “Control Panel/System/Device Manager”.

Ultra DMA/33 Hard Disk Protocol:

Ultra DMA is a new protocol for the ATA/IDE hard disk drive interface that doubles the current burst data transfer rate to 33 megabytes (MB) per second.  The protocol is a Quantum Corporation patented technology that will be implemented in industry standard hard drive  products.  The TX531 provides a utility disk to install the Ultra DMA/33 capability onto the system to support hard disk with Ultra DMA/33 mode.

1.4    System Performance

The following CPU benchmark programs provide a wide range of performance indexes, which do not reveal the true speed that the motherboard can provide.

This test is performed on most types of CPUs which are installed on the TX531 motherboard.

| SOFTWARE CPU TYPE | LANDMARK V2.0 | POWER METER V1.8 MIPS | NORTON V8.0 CPU SPEED |
| --- | --- | --- | --- |
| PENTIUM 133 | 771.33 MHz | 67.7 MIPS | 423.5 |
| PENTIUM 150 | 878.06 MHz | 74.8 MIPS | 482.1 |
| PENTIUM 166 | 964.19 MHz | 83.6 MIPS | 529.3 |
| PENTIUM 200 | 1157.04 MHz | 98.1 MIPS | 635.2 |
| PENTIUM MMX 166 | 1093.16 MHz | 83.6 MIPS | 567.2 |
| PENTIUM MMX 200 | 1311.8 MHz | 101.6 MIPS | 680.6 |
| PENTIUM MMX 233 | 1530.42 MHz | 113.8 MIPS | 784.0 |
| CYRIX 6x86-133+ | 1353.98 MHz | 74.8 MIPS | 761.2 |
| CYRIX 6x86-150+ | 1469.25 MHz | 81.3 MIPS | 826 |
| CYRIX 6x86-166+ | 1613.31 MHz | 88.9 MIPS | 907 |
| CYRIX 6x86-200+ | 1089 MHz | 101.6 MIPS | 1018.1 |
| AMD K5-133 | 892.59 MHz | 66.1 MIPS | 430.5 |
| AMD K5-166 | 1041.38 MHz | 76.9 MIPS | 502.3 |
| AMD K6-166 | 1687.6 MHz | 123.6 MIPS | 991.9 |
| AMD K6-200 | 2025.15 MHz | 148.7 MIPS | 1190.3 |
| AMD K6-233 | 2362.67 MHz | 177.7 MIPS | 1388.7 |

Chapter  2    Installation

2.1    Installing DRAM

The hardware must be set up for a variety of functions before the system is ready to operate.  It is easy to set up the TX531 motherboard.  The user only has to adjust a few jumpers, connectors, and sockets.

The TX531 motherboard can support extended memory of 8MB to 256MB.

n  The layout of the DRAM memory banks on board is shown below:

SIMM4   SIMM3  SIMM2  SIMM1

BANK1  BANK0

BANK1               BANK0

SIMM1+SIMM2

TOTAL

SIMM3+SIMM4

and/or                                 =

TOTAL

and/or

DIMM1

DIMM2

=

System Memory (SIMM & DIMM)

This motherboard supports four 72-pin, 32-bit SIMMs (Single Inline Memory Modules) of 4, 8, 16, 32, or 64 MB to form a memory size from 8MB to 256MB.  The SIMMs can be either 60ns or 70ns Fast Page Mode (FPM) (Asymmetric or Symmetric), or Enhanced Data Out (EDO) (BEDO & Parity are not supported).  SIMMs must be installed in pairs so that each Row (see Map of Motherboard for Row locations) contains 64-bits of the same size and type of memory chips.  One side (with memory chips) of the SIMM module takes up half a Row on the motherboard.

Dual Inline Memory Modules (DIMMs) can be used when the SIMM sockets are not used.  Two sockets are available for 3.3 Volt (power level) Unbuffered Synchronous DRAMs (SDRAM) or EDO DRAM of either 8, 16, 32, 64, or 128MB to form a memory size between 8MB to 256MB.  One side (with memory chips) of the DIMM module takes up one Row on the motherboard.

n   Table 1 (SIMM)

| BANK 0 |  | BANK 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SIMM1 | SIMM2 | SIMM3 | SIMM4 |  | Total Memory |
| 4MB | 4MB | None | None |  | 8MB |
| 4MB | 4MB | 4MB | 4MB |  | 16MB |
| 8MB | 8MB | None | None |  | 16MB |
| 4MB | 4MB | 8MB | 8MB |  | 24MB |
| 8MB | 8MB | 4MB | 4MB |  | 24MB |
| 8MB | 8MB | 8MB | 8MB |  | 32MB |
| 16MB | 16MB | None | None |  | 32MB |
| 4MB | 4MB | 16MB | 16MB |  | 40MB |
| 16MB | 16MB | 4MB | 4MB |  | 40MB |
| 8MB | 8MB | 16MB | 16MB |  | 48MB |
| 16MB | 16MB | 8MB | 8MB |  | 48MB |
| 16MB | 16MB | 16MB | 16MB |  | 64MB |
| 32MB | 32MB | None | None |  | 64MB |
| 4MB | 4MB | 32MB | 32MB |  | 72MB |
| 32MB | 32MB | 4MB | 4MB |  | 72MB |
| 8MB | 8MB | 32MB | 32MB |  | 80MB |
| 32MB | 32MB | 8MB | 8MB |  | 80MB |
| 16MB | 16MB | 32MB | 32MB |  | 96MB |
| 32MB | 32MB | 16MB | 16MB |  | 96MB |
| 32MB | 32MB | 32MB | 32MB |  | 128MB |

n   Table 2  (DIMM)

| DIMM1 (BANK0) | DIMM2 (BANK1) | Total Memory |
| --- | --- | --- |
| 8MB | --- | 8MB |
| --- | 8MB | 8MB |
| 8MB | 8MB | 16MB |
| 16MB | 16MB | 32MB |
| 32MB | 32MB | 64MB |
| 64MB | 64MB | 128MB |

Note: DIMM and SIMM can’t be installed at the same time.

2.2   TX531 Board Layout

The general layout of the TX531 motherboard is shown in the following diagram:

COMA

1

4

U4

5            8

1            4

FDC

33     34

U3

1                  3

J3

JP2

KB1

DIMM2

DIMM1

1        2

PRN

1      14

13     26

39     40

39     40

1        2

1        2

IDE2

IDE1

PCI4

PCI3

PCI2

PCI1

JP5

BT1

32Kx32/64

U9

JP1

USB

U14

32Kx32/64

SIMM1

SIMM2

SIMM3

SIMM4

COMB

CN1

CN2

-                        +

32Kx8

U15

JP6

1     2

7    8

J14

KEYLOCK

1                5

1            4

J13

SPEAKER

J12  J11   J10

RESET HLED   SPLED

1       2

1       2

9    10

9    10

1                  3

JP3

J9

CPUFAN

J8

PWBTN

SL5

SL3

SL1

U5

SL6

SL4

SL2

U8

SW1

Acer

M1543 AO

JP8

U11

Acer

M1543 B1

CPU SOCKET 7

2.3	Setting Other Jumpers & Connectors

Connector Description

| Connector | Pin Out | Signal Name |
| --- | --- | --- |
| JP1 USB CONNECTOR | 1 2 3 4 | +5V DC ( RED WIRE) DATA OUT DATA OUT GROUND |
| J12 RESET | 1 2 | GROUND RESET IN |
| J14 KEYLOCK CONNECTOR | 1 2 3 4 5 | +5V DC NC GROUND KEYBOARD INHIBITOR GROUND |
| J13 SPEAKER CONNECTOR | 1 2 3 4 | DATA LINE NC GROUND +5V DC |
| J11 HDD_LED | 1 2 | HARD DISK LED |
| J9 CPU FAN HEADER | 1 2 | +12V GROUND |
| l l ¡ ¡ JP2 CLEAR CMOS | 1 2 | CLEAR CMOS NORMAL |
| J10 SPEED LED | 1 2 | TURBO LED |
| J8 SMI | 1 2 | Suspend Switch |

DIMMs memory voltage selection

| 5V EDO type | § 3V SDRAM (Default) |
| --- | --- |
| JP6 ¡ ¡ ¡ ¡ l l l l 7 8 1 2 ¡ ¡ ¡ ¡ l l l l 7 8 1 2 JP6 |  |

§ Default Setting

- LED Lead (TB LED)

- This indicates whether a message has been received from a fax/modem.  The LED will remain lit when there is no signal and blink when there is data transfer or waiting in the inbox.

- Reset Switch Lead (RESET)

- This 2-pin connector connects to the case-mounted reset switch for rebooting your computer without having to turn off your power switch.  This is a preferred method of rebooting in order to prolong the life of the system’s power supply.

- Keyboard Lock Switch Led & System Power LED (KEYLOCK)

- This 5-pin connector connects to the case-mounted keyboard lock switch for locking the keyboard and also to connect the system power LED.  The system power LED lights when the system is powered on and blinks in sleep mode.

- Speaker Connector (SPEAKER)

- This 4-pin connector connects to the case-mounted speaker.

- PS/2 Mouse, USB, Infrared Connector

- If you are using a PS/2 mouse, USB, or Infrared devices, you must purchase an optional external connector set.  The external connector set connects to the 18 pin block and mounts to an open slot on your computer’s chassis.  The system will direct IRQ12 to the PS/2 mouse if one is detected.  If not detected, expansion cards can use IRQ12.  See ”PS/2 Mouse Control” in BIOS Features Setup and “USB Function” in PnP and PCI Setup of the BIOS SOFTWARE.  See “Second Infrared” connector for details on the infrared connector.

- Second Infrared (IrDA) & Fast IR-Compliant Infrared Connector (IR)

- This is a second connector that supports the optional wireless transmitting and receiving infrared module.  This module mounts to a small opening on system cases that support this feature.  You must also configure the setting through “UART2 Use Infrared” in Chipset Features Setup to select whether UART2 is directed for use with COM2 or IrDA.  Use the five pins as shown on the Back View and connect a ribbon cable from the module to the motherboard according to the pin definitions.

- SMI Suspend Switch Lead (SMI function)

This allows the user to manually place the system into a suspend mode or “Green” mode where system activity will be instantly decreased to save electricity and expand the life of certain components when the system is not in use.  This 2-pin connector (see the figure below) connects to the case-mounted suspend switch.  If you do not have a switch for the connector,  you may use the “Turbo Switch” since it does not have a function.  SMI is activated when it detects a Short to open moment and therefore leaving it shorted will not cause any problems.  May require one or two pushes depending on the position of the switch .  Wake-up can be controlled by settings in the BIOS but the keyboard will always allow wake-up (the SMI lead cannot wake-up the system).  If you want to use this connector, “Suspend Switch” in the Power Management Setup of the BIOS SOFTWARE section should be on the default setting of Enable.

2.4	Setting CPU Configuration

Definition of DIP Switch SW 1

SW 1,2,3 - CPU Ratio (1.5x, 2.0x, 2.5x, 3.0x, 3.5x same as 1.5x setting ).

SW 4,5,6 - CPU Frequency (50/55/60/66/75 MHz clock).

For JP8: CPU Mode type:

“OFF” Position: Interleave Burst (Intel/AMD CPUs including MMX).

“ON” Position: Linear Burst (Cyrix CPU including 6x86MX Processor).

- TX531 CPU Auto-Voltage Detection Feature

The TX531 provides Auto-detection voltage for all series of Pentium-based Processor.  No setting is required to prevent wrong CPU voltage setting.  All Pentium-based processor use two power supplies; one supply is for the CPU core, and the other is for the I/O interface.  Some CPU vendor provide the same voltage for the core and I/O, it is called “SINGLE VOLTAGE CPU”.  If the core and I/O use different voltages, it is called “DUAL VOLTAGES CPU”.  Instead of setting the CPU as “single” or “dual” voltage by jumper, TX531 Pentium-based series will automatic detect the CPU is “single” or “dual” voltage when system power-on.

Processors Configuration

| CPU Type | Jumper Setting |
| --- | --- |
| SW1 PENTIUM 90MHz/AMD K5-PR90/ AMD K5-PR120 |  |
| SW1 PENTIUM 100MHz/AMD K5-PR100/ AMD K5-PR133 |  |
| SW1 PENTIUM 120MHz |  |
| SW1 PENTIUM 133MHz |  |
| SW1 PENTIUM 150MHz |  |
| SW1 PENTIUM 166MHz/PENTIUM MMX-166/ AMD K5-PR166/AMD K6-PR166 |  |

(Continued)

Processors Configuration

| CPU Type | Jumper Setting |
| --- | --- |
| SW1 PENTIUM 166MHz/PENTIUM MMX-166/ AMD K5-PR166/AMD K6-PR166 |  |
| SW1 PENTIUM 200MHz/PENTIUM MMX-200/ AMD K6-PR200 |  |
| SW1 PENTIUM MMX-233MHz/ AMD K6-PR233 |  |
| SW1 CYRIX 6x86-133+ |  |
| SW1 CYRIX 6x86-150+ |  |
| SW1 JP8 · · JP8 · · JP8 · · CYRIX 6x86-166+ |  |

(Continued)

Processors Configuration

| CPU Type | Jumper Setting |
| --- | --- |
| SW1 CYRIX 6x86-200+ |  |
| SW1 CYRIX 6x86MX-PR166 |  |
| SW1 CYRIX 6x6MX-PR200 |  |
| SW1 JP8 · · JP8 · · JP8 · · JP8 · · CYRIX 6x86MX-PR233 |  |

Manual Configuration for Switch CPU speed bus & CPU ratio relationships of the CPUs:

Definition of DIP Switch

DIP: 1,2,3 - CPU Ratio (1.5x,2.0x,2.5x,3.0x,3.5x same as 1.5x setting).

DIP: 4,5,6 - CPU Frequency (50/55/60/66/75 Mhz clock).

| DIP1 | DIP2 | DIP3 | Multiplier |
| --- | --- | --- | --- |
| ON | ON | OFF | 2.5x |
| OFF | ON | OFF | 3.0x |
| ON | OFF | OFF | 2.0x |
| OFF | OFF | OFF | 1.5/3.5x |

| DIP4 | DIP5 | DIP6 | CPU Clock (MHz) |
| --- | --- | --- | --- |
| ON | ON | ON | 50 |
| ON | ON | OFF | 55 |
| ON | OFF | OFF | 60 |
| OFF | OFF | OFF | 66 |
| OFF | ON | ON | 75 |

2.5	Installing FLASH ROM

JP3

1                  3

9    10

9    10

1       2

1       2

-                        +

U4

CN2

CN1

COMB

COMA

JP1

USB

BT1

PCI1

PCI2

PCI3

PCI4

IDE1

IDE2

1        2

1        2

39     40

39     40

13     26

1      14

PRN

1        2

KB1

JP2

J3

1

3

U3

33     34

FDC

5            8

1            4

| § 5V FLASH ROM | 12V FLASH ROM |
| --- | --- |
| ¡ l l l 1 2 3 JP3 l l ¡ 1 2 3 JP3 |  |

§ Default Setting

2.6	Installing Flash ROM Utility

Please check the utility software included with your motherboard.  This is the Flash Memory writer utility that updates the BIOS by uploading a new BIOS file to the programmable Flash ROM chip on the motherboard.

Flash ROM Writer Utility AMIFLASH.COM

AMIFLASH Version 5.30 - Flash EPROM Programming Utility Copyright (C) 1992-1996 American Megatrends Inc. Release Date : 05/12/96

Help/Error Message Enter BIOS Filename:

Press <ESC> to Exit Enter the BIOS Filename from which Flash EPROM will be programmed. The Format:- [Drv:\Pathname\Filename.Ext] The Filename must end with a <ENTER>.

You cannot run this utility from the Windows 95 MS-DOS box.  Please run it in real mode.  The method is :

- Restart your computer.

- Press F8 to show the multiboot menu as soon as the “Starting Windows 95...” message appears on the screen.

- Choose 6 (Command prompt only).

- Run “AMIFLASH” to write the updated content to your BIOS if necessary.

Another method is: Create a bootable system floppy diskette by typing “FORMAT A:/S” from the DOS environment without creating AUTOEXEC.BAT and CONFIG.SYS, then save the AMIFLASH utility and the BIOS file to the diskette.  Note that you should have a BIOS file ready to update your BIOS chip if you really want to change the original content of your BIOS.

Notice : If the flashing of the BIOS content fails to boot-up, we recommend you to refer to your nearest Technical support site for replacement or have a qualified technician to do the updates for your system.

Chapter  3    System BIOS Setup

This chapter discusses AMIBIOS Setup program built into the ROM BIOS.  The Setup program allows users to modify the basic system configuration.  This special information is then stored into battery-backed RAM so that it contains the Setup program when the power is turned off.

The AMIBIOS has been customized by adding important, but non-standard features such as virus and password protection as will as special support for detailed fine-tuning of the chipset controlling the entire system.

The Setup utility provided by AMIBIOS is accessed by pressing <Del> at the appropriate time during system boot.  Setup configures data in CMOS RAM.

The following screen shows the types of AMIBIOS Setup:

| AMIBIOS HIFLEX SETUP UTILITY (C)1996 American Megatrends, Inc. All Rights Reserved |  |  |
| --- | --- | --- |
|  | Standard CMOS Setup |  |
| Advanced CMOS Setup |  |  |
| Advanced Chipset Setup |  |  |
| Power Management Setup |  |  |
| PCI / Plug and Play Setup |  |  |
| Peripheral Setup |  |  |
| Auto-Detect Hard Disks |  |  |
| Change User Password |  |  |
| Change Supervisor Password |  |  |
| Change Language Setting |  |  |
| Auto Configuration with Optimal Settings |  |  |
| Auto Configuration with Fail Safe Settings |  |  |
| Save Settings and Exit |  |  |
| Exit Without Saving |  |  |
| Standard CMOS setup for changeing time, date, hard disk type, etc. |  |  |
| ESC: Exit áâ: Sel F2/F3: Color F10: Save & Exit |  |  |

3.1	Standard CMOS Setup

The AMIBIOS Setup options described in this section are displayed by choosing the Standard Setup icon from the Setup section on the AMIBIOS Setup main menu.  All Standard Setup options are described in this section.

- Date/Time

Select the Date/Time option to change the date or time.  The current date and time are displayed.  Enter new values through the displayed window.

- Floppy Drive A, B

Choose the Floppy Drive A or B icon to specify the floppy drive type.  The settings are 360 KB 5¼”, 1.2 MB 5¼”, 720 KB 3½”, 1.44 MB 3½”, or 2.88 MB 3½”.

- IDE Device Auto-detection for Primary/Secondary Hard Disk/CD-ROM Drives/LS-120 device.

- Configuring IDE Drives

If the hard disk drive to be configured is an IDE drive, select the appropriate drive icon (Pri Master, Pri Slave, Sec Master, or Sec Slave).  Choose the Type parameter and select Auto.

AMIBIOS automatically detects the IDE drive parameters and displays them.  Click on the OK button to accept these parameters.

Click on LBA/Large Mode and choose On to enable support for IDE drives with capacities greater than 528 MB.  Note that hard disk with LBA mode formatted on other system may not bootup and you needs to do a “sys c:” for system bootup.

Click on Block Mode and choose On to support IDE drives that use Block Mode.

Click on 32Bit Mode and choose On to support IDE drives that permit 32-bit accesses.

- Configuring a CD-ROM Drive

Select the appropriate drive icon (Pri Master, Pri Slave, Sec Master, or Sec Slave). Choose the Type parameter and select CD ROM.  You can boot the computer from a CD-ROM drive.

- Configuring a LS-120 device

The LS-120 device will be auto-detected by BIOS.  To create LS-120 device boot able Disc, format the LS-120 disk under Windows 95 or for data storage.  Do not format LS-120 disk under DOS environment.

- Anti-Virus Option

Do not enable this option during operating system installation like OS/2 and Windows 95.  This may render your system  unable to install.

3.2	Advanced CMOS Setup

The AMIBIOS Setup options described in this section are displayed by choosing the Advanced Setup icon from the Setup section on the AMIBIOS Setup main menu.  All Advanced Setup options are described in this section.

| AMIBIOS SETUP —– ADVANCED CMOS SETUP (C)1996 American Megatrends, Inc. All Rights Reserved |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1st Boot Device | FLOPPY |  | Available Options: |  |
| 2nd Boot Device | IDE-0 |  |  | Disabled |
| 3rd Boot Device | CDROM |  |  | FLOPTICAL |
| 4th Boot Device | Disabled |  |  |  |
| S.M.A.R.T. for Hard Disks | Disabled |  |  |  |
| Quick Boot | Disabled |  |  |  |
| BootUp Num-Lock | Off |  |  |  |
| Floppy Driver Swap | Disabled |  |  |  |
| Floppy Driver Seek | Disabled |  |  |  |
| PS/2 Mouse Support | Disabled |  |  |  |
| System Keyboard | Absent |  |  |  |
| Primary Display | Absent |  |  |  |
| Password Check | Setup |  |  |  |
| Parity Check | Disabled |  |  |  |
| Boot To OS/2 | No |  |  |  |
| Wait For “F1” If Error | Disabled |  |  |  |
| Hit “DEL” Message Display | Disabled |  | ESC: Exit áâ : Sel |  |
| External Cache | Disabled |  | PgUp/PgDn : Modify |  |
| System BIOS Cacheable | Enabled |  | F2/F3 : Color |  |

- xxx Boot Device

Piority of Device pre-defined in-sequence for Booting.

- S.M.A.R.T. Feature

S.M.A.R.T. (Self-Monitoring, Analysis and Reporting Technology) is a technology developed to manage the reliability of the hard disks by predicting some (but NOT ALL) of the future device failures.  This feature helps BIOS warn the user of the possible device failure thereby giving user a chance to back up the device and replace the device before actual failure happens.  S.M.A.R.T. capable devices should predict an impending failure and return that information through the Return S.M.A.R.T. Status command.  Note that S.M.A.R.T. can not predict all future device failures and it should be used as an warning tool, not as a tool to predict the device reliability.

- Quick Boot

Set this option to Enabled to permit AMIBIOS to boot within 5 seconds.  This option replaces the old Above 1 MB Memory Test option.  The Optimal and Fail-Safe default settings are Disabled.

| Setting | Description |
| --- | --- |
| Disabled | AMIBIOS tests all system memory. AMIBIOS wait up to 40 seconds for a READY signal from the IDE drive. AMIBIOS waits up to .5 seconds after sending a RESET signal to the IDE drive to permit the IDE drive to send a READY signal. AMIBIOS checks if the <Del> key was pressed. AMIBIOS Setup is executed if <Del> was pressed. |
| Enabled | AMIBIOS does not test any system memory above 1 MB. AMIBIOS does not wait for a READY signal from the IDE drive. If a READY signal is not received immediately, AMIBIOS does not configure the drive. AMIBIOS does not wait .5 seconds after sending a RESET signal to the IDE drive. AMIBIOS does not check if the <Del> key was pressed. You cannot run AMIBIOS Setup at system boot if this setting is selected. |

- Boot Up Sequence

This option sets the sequence of boot drives (floppy drive A:, hard disk drive C:, or a CD-ROM drive) that AMIBIOS attempts to boot from after AMIBIOS POST completes.  The settings are C:,A:, CDROM, A:,C:, CDROM, or CDROM, C:,A:. The Optimal and Fail-Safe default settings are C:,A:,CDROM.

- Boot Up CPU Speed

This option sets the CPU speed when the computer boots.  The settings are Low or High.  The Optimal and Fail-Safe default settings are High.

- Boot Up Num Lock

When this option is set to On, AMIBIOS turns off the Num Lock key when the system is powered on so the end user can use the arrow keys on both the numeric keypad and the keyboard.  The settings are On or Off.  The Optimal default and Fail-Safe default settings are On.

- Floppy Drive Swap

Set this option to Enabled to specify that floppy drives A: and B: are swapped.  The settings are Enabled or Disabled.  The Optimal and Fail-Safe default settings are Disabled.

- Floppy Drive Seek

When this option is set to Enabled, AMIBIOS performs a Seek command on floppy drive A: before booting the system.  The settings are Enabled or Disabled.  The Optimal default setting is Disabled.  The Fail-Safe default setting is Enabled.

- PS/2 Mouse Support

When this option is set to Enabled, AMIBIOS supports a PS/2-type mouse.  The settings are Enabled or Disabled.  The Optimal and Fail-Safe default settings are Disabled.

- System Keyboard

This option configures the keyboard.  The settings are Present or Absent.  If you select Absent, AMIBIOS does not report keyboard errors.  The Optimal and Fail-Safe default settings are Present.

- Primary Display

This option configures the primary display subsystem in the computer.  The settings are Absent, Mono (monochrome), 40CGA, 80CGA, or VGA/EGA.  The Optimal and Fail-Safe default settings are VGA/EGA.

- Password Check

This option specifies the type of AMIBIOS password protection that is implemented. The Optimal and Fail-Safe default settings are Setup.  The settings are:

| Setting | Description |
| --- | --- |
| Setup | The password prompt appears only when an end user attempts to run AMIBIOS Setup. |
| Always | A password prompt appears every time the computer is powered on or rebooted. |

The AMIBIOS password does not have to be enabled. The end user sets the password by choosing the Password icon on the AMIBIOS Setup screen.

- Boot to OS/2

Set this option to Yes to permit AMIBIOS to run properly if OS/2 or any other operating system that does not support Plug and Play is to be run on this computer.  If running  OS/2 in a computer with 64 MB or more of system memory, set this option to Yes.  The settings are Yes or No.  The Optimal and Fail-Safe default settings are No.

Wait for "F1" If Error

AMIBIOS POST runs system diagnostic tests that can generate a message followed by:

Press <F1> to continue

If this option is set to Enabled, AMIBIOS waits for the end user to press <F1> before continuing.  If this option is set to Disabled, AMIBIOS continues the boot process without waiting for <F1> to be pressed.  The settings are Enabled or Disabled.  The Optimal default and Fail-Safe default settings are Enabled.

Hit "DEL" Message Display

Disabling this option prevents

Hit <DEL> if you want to run Setup

from appearing when the system boots.  The settings are Enabled or Disabled.  The Optimal and Fail-Safe default settings are Enabled.

- External Cache

This option selects the type of caching algorithm used by AMIBIOS and the CPU to access L2 secondary (external) cache memory.  The settings are:

| Setting | Description |
| --- | --- |
| WriteBack | A write-back algorithm is used. |
| WriteThru | A write-through algorithm is used. |
| Disabled | AMIBIOS does not specify the type of caching algorithm. |

The Optimal default setting is WriteBack. The Fail-Safe default setting is Disabled.

- System BIOS Cacheable

AMIBIOS always copies the system BIOS from ROM to RAM for faster execution. Set this option to Enabled to permit the contents of the F0000h RAM memory segment to be written to and read from cache memory.  The settings are Enabled or Disabled.  The Optimal default setting is Enabled.  The Fail-Safe default setting is Disabled.

C000,16K Shadow

C400,16K Shadow

These options specify how the contents of the video ROM at C0000h are handled. The settings are:

| Setting | Description |
| --- | --- |
| Disabled | The Video ROM is not copied to RAM. |
| Cached | The contents of the video ROM area from C0000h - C7FFFh are not only copied from ROM to RAM, the contents of the C0000h - C7FFFh RAM area can be written to or read from cache memory. |
| Enabled | The contents of the video ROM area from C0000h - C7FFFh are copied (shadowed) from ROM to RAM for faster execution. |

The Optimal and Fail-Safe default settings are Cached.

C800,16K Shadow

CC00,16K Shadow

D000,16K Shadow

D400,16K Shadow

D800,16K Shadow

DC00,16K Shadow

These options specify how the contents of the adapter ROM named in the option title are handled. The ROM area that is not used by ISA adapter cards will be allocated to PCI adapter cards.  The settings are:

| Setting | Description |
| --- | --- |
| Disabled | The specified ROM is not copied to RAM. |
| Cached | The contents of the ROM area are not only copied from ROM to RAM for faster execution, the contents of the RAM area can be written to or read from cache memory. |
| Enabled | The contents of the ROM area are copied from ROM to RAM for faster execution. |

The Optimal and Fail-Safe default settings are Cached.

3.3	 Advanced Chipset Setup

Choose the Chipset Setup icon from the Setup section on the AMIBIOS Setup main menu.  All Chipset Setup options are then displayed and are described in this chapter.

| AMIBIOS SETUP —– ADVANCED CHIPSET SETUP (C)1996 American Megatrends, Inc. All Rights Reserved |  |  |  |
| --- | --- | --- | --- |
| USB Function | Disabled |  | Available Options: |
| USB Keyboard/Mouse Legacy Support | Enabled |  | Disabled |
| DRAM Write Timing | 4-3-3-3 |  | Enabled |
| Page Mode DRAM Read Timing | X-4-4-4 |  |  |
| EDO DRAM Read Timing | X-3-3-3 |  |  |
| DRAM Data Integrity Mode | Parity |  |  |
| AT Bus Clock | Auto |  |  |
| Cyrix Burst Mode | Linear |  |  |
| Memory Hole | Disabled |  |  |
|  |  |  | ESC: Exit áâ: Sel |
|  |  |  | PgUp/PgDn: Modify |
|  |  |  | F2/F3: Color |

- USB Function

Set this option to Enabled to enable the system BIOS USB (Universal Serial Bus) functions.  The settings are Enabled or Disabled.  The Optimal and Fail-Safe default settings are Disabled.

- USB Keyboard/Mouse Legacy Support

Set this option to Enabled to provide support for older non-USB keyboard and mouses.  The settings are Enabled or Disabled.  The Optimal and Fail-Safe default settings are Disabled.

- DRAM Write Timing

This option specifies the access timings for DRAM system memory write operations.  The Optimal and Fail-Safe default settings are x4-4-4 or x-3-3-3.  Default setting as x-4-4-4.

- Page Mode DRAM Read Timing

This option specifies the access timings for Page Mode DRAM system memory read operations.  The settings are x-4-4-4 or x-3-3-3. The Optimal default setting as x-4-4-4.  It is recommend that for CPU Frequency 233 MHz or CPU clock above 66MHz speed, it should be set as x-4-4-4.

- EDO DRAM Read Timing

This option specifies the access timing for Page Mode DRAM system memory read operations.  The settings are x-4-4-4-4, or x-3-3-3.  The Optimal default settings are x-3-3-3.

- DRAM Data Integrity Mode

This option sets the Error Check and Correction feature (ECC) (SIMMs with Parity RAM) to provides single-error correction, double-error correction and detection of all errors confined to a single nibble for the DRAM data integrity.  The settings are Parity/ECC.  The Optimal and Fail-Safe default settings are Parity.  If you do not install SIMMs with Parity Check RAM, then don’t care or “Parity”.  Do not set it as “ECC” option if your SIMMs do not have parity check RAM as wrong setting may cause system unstable.

- AT Bus Clock

This option set the AT Bus Clock for system I.O.  Default setting set as “Auto”.

- Cyrix Burst Mode

This option enable the Cyrix CPU to take advantage of the Burst mode incorporated in the CPU to increase performance.

- Memory Hole

This option allows the end user to specify the location of a memory hole for ISA card with firm ware ROM.  The settings are Disabled, 512-640K, 14-16 MB, or 15-16M (from 15MB to 16 MB).  The Optimal and Fail-Safe default settings re Disabled.

Memory Specification Notice

The problem with EDO RAM when looking in the future is that it hardly works with any bus speed higher than 66 MHz, which is already reached.  As CPUs demand higher bus speeds and the Cyrix 6x86 already needing 75 MHz, it is not recommended to use EDO Memory on the TX531 for system clock frequency of above 66 MHz or install SDRAM DIMMs memory which supports up to 83.3 MHz bandwidth depending on the type of the SDRAM DIMMs installed.  Current Intel and AMD CPUs (100~233 MHz) require 66 MHz system clock.

3.4	Power Management Setup

The AMIBIOS Setup options described in this section are selected by choosing the Power Mgmt Setup icon from the Setup section on the AMIBIOS Setup main menu.

| AMIBIOS SETUP —– POWER MANAGEMENT SETUP (C)1996 American Megatrends, Inc. All Rights Reserved |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Power Management/APM |  | Disabled |  | Available Options: |  |
| Green PC Monitor Power State |  | Stand By |  |  | Disabled |
| Video Power Down Mode |  | Disabled |  |  | Enabled |
| Hard Disk Power Down Mode |  | Disabled |  |  |  |
| Standby Time out |  | Disabled |  |  |  |
| Suspend Time Out |  | Disabled |  |  |  |
|  | Monitor Parallel Port | Yes |  |  |  |
|  | Monitor Serial Port | Yes |  |  |  |
|  | Monitor Floppy | Yes |  |  |  |
|  | Monitor VGA | No |  |  |  |
|  | Monitor Pri-HDD | No |  |  |  |
|  | Monitor Sec-HDD | No |  |  |  |
| Power Button Function |  | Green |  |  |  |
|  |  |  |  | ESC : Exit áâ : Sel |  |
|  |  |  |  | PgUp/PgDn: Modify |  |
|  |  |  |  | F2/F3: Color |  |

- Power Management/APM

Set this option to Enabled to enable the Intel 430TX power management features and APM (Advanced Power Management).  The settings are Enabled or Disabled. The Optimal and Fail-Safe default settings are Enabled.

- Green PC Monitor Power State

This option specifies the power state that the green PC-compliant video monitor enters when AMIBIOS places it in a power savings state after the specified period of display inactivity has expired.  The settings are Off, Standby, Suspend, or Disabled. The Optimal and Fail-Safe default settings are Standby.

- Video Power Down Mode

This option specifies the power conserving state that the VESA VGA video subsystem enters after the specified period of display inactivity has expired.  The settings are Disabled, Standby, or Suspend. The Optimal and Fail-Safe default settings are Disabled.

- Hard Disk Power Down Mode

This option specifies the power conserving state that the hard disk drive enters after the specified period of hard drive inactivity has expired.  The settings are Disabled, Standby, or Suspend.  The Optimal and Fail-Safe default settings are Disabled.

- Standby Time out

This option specifies the length of a period of system inactivity while in Full power on state.  When this length of time expires, the computer enters Standby power state.  The settings are Disabled, 1 min, 2 min, 3 min, 4 min, 5 min, 6 min, 7 min, 8 min, 9 min, 10 min, 11 min, 12 min, 13 min, 14 min, or 15 min. The Optimal and Fail-Safe default settings are Disabled.

- Suspend Time out

This option specifies the length of a period of system inactivity while in Standby state.  When this length of time expires, the computer enters Suspend power state.  The settings are Disabled, 1 min, 2 min, 3 min, 4 min, 5 min, 6 min, 7 min, 8 min, 9 min, 10 min, 11 min, 12 min, 13 min, 14 min, or 15 min. The Optimal and Fail-Safe default settings are Disabled.

- Power Button Function (SMI switch)

This option specifies the length of a period of system inactivity while in Standby.

3.5	PCI/PnP Setup

Choose the PCI/PnP Setup icon from the AMIBIOS Setup screen to display the PCI and Plug and Play Setup options, described below.

| AMIBIOS SETUP —– PCI / PLUG PLAY SETUP (C)1996 American Megatrends, Inc. All Rights Reserved |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OnChip USB | Disabled |  |  | Available Options: |  |
| Plug and Play Aware O/S | No |  |  |  | Disabled |
| PCI IDE BusMaster | Disabled |  |  |  | Enabled |
| Assign IRQ To PCI VGA Cars | No |  |  |  |  |
| IRQ3 | PCI/PnP |  |  |  |  |
| IRQ4 | PCI/PnP |  |  |  |  |
| IRQ5 | PCI/PnP |  |  |  |  |
| IRQ7 | PCI/PnP |  |  |  |  |
| IRQ9 | PCI/PnP |  |  |  |  |
| IRQ10 | PCI/PnP |  |  |  |  |
| IRQ11 | PCI/PnP |  |  |  |  |
| IRQ12 | PCI/PnP |  |  |  |  |
| IRQ14 | PCI/PnP |  |  |  |  |
| IRQ15 | PCI/PnP |  |  |  |  |
| Reserved Memory Size | Disabled |  |  |  |  |
| Reserved Memory Address | C8000 |  |  |  |  |
|  |  |  |  | ESC : Exit áâ : Sel |  |
|  |  |  | PgUp/PgDn: Modify |  |  |
|  |  |  | F2/F3 : Color |  |  |

- OnChip USB:

Set this option to Enabled if any USB device is installed.  Default setting as Disabled.

- Plug and Play-Aware O/S

Set this option to Yes if the operating system in this computer is aware of and follows the Plug and Play specification. Currently, only Windows 95 is PnP-aware. The settings are Yes or No.  The Optimal and Fail-Safe default settings No.

- Reserved Memory Size

This option specifies the size of the memory area reserved for legacy ISA adapter cards.  The settings are Disabled, 16K, 32K, or 64K.  The Optimal and Fail-Safe default settings are Disabled.

- Reserved Memory Address

This option specifies the beginning address (in hex) of the reserved memory area.  The specified ROM memory area is reserved for use by legacy ISA adapter cards.

The settings are C0000, C4000, C8000, CC000, D0000, D4000, D8000, or DC000. The Optimal and Fail-Safe default settings are C8000.

3.6	Peripheral Setup

Choose the Peripheral Setup icon from the AMIBIOS Setup screen to display the Peripheral Setup options, described below.

| AMIBIOS SETUP —– PERIPHERAL SETUP (C)1996 American Megatrends, Inc. All Rights Reserved |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| OnBoard FDC |  | Auto |  |  | Available Options: |  |
| OnBoard Serial Port1 |  | 3F8h |  |  |  | Auto |
| Serial Port1 Mode |  | Normal |  |  |  | Disabled |
| Serial Port1 IRQ |  | 4 |  |  |  | Enabled |
|  | IR Transmitter Polarity | N/A |  |  |  |  |
|  | IR Receiver Polarity | N/A |  |  |  |  |
|  | IR Half-Duplex Time-Out | N/A |  |  |  |  |
| OnBoard Serial Port2 |  | 2F8h |  |  |  |  |
| Serial Port2 Mode |  | Normal |  |  |  |  |
| Serial Port2 IRQ |  | 3 |  |  |  |  |
|  | IR Transmitter Polarity | N/A |  |  |  |  |
|  | IR Receiver Polarity | N/A |  |  |  |  |
|  | IR Half-Duplex Time-Out | N/A |  |  |  |  |
| OnBoard Parallel Port |  | 378 |  |  |  |  |
| Parallel Port Mode |  | Normal |  |  |  |  |
| EPP Version |  | N/A |  |  |  |  |
| Parallel Port IRQ |  | 7 |  |  |  |  |
| Parallel Port DMA Channel |  | N/A |  |  |  |  |
| Onboard IDE |  | Both |  |  |  |  |
|  |  |  |  | ESC : Exit áâ : Sel |  |  |
|  |  |  |  | PgUp/PgDn : Modify |  |  |
|  |  |  |  | F2/F3 : Color |  |  |

- Onboard FDC

This option enables the FDC (Floppy Drive Controller) on the motherboard.  The settings are Auto (AMIBIOS automatically determines if the floppy controller should be enabled), Enabled, or Disabled.  The Optimal and Fail-Safe default settings are Auto.

- Onboard Serial Port1

This option specifies the base I/O port address of serial port 1.  The settings are Auto (AMIBIOS automatically determines the correct base I/O port address), Disabled, 3F8h, 2F8h, 2E8h, or 3E8h.  The Optimal and Fail-Safe defaults are 3F8H.

- Onboard Serial Port2

This option specifies the base I/O port address of serial port 2.  The settings are Auto (AMIBIOS automatically determines the correct base I/O port address), Disabled, 3F8h, 2F8h, 2E8h, or 3E8h.  The Optimal and Fail-Safe defaults are 2F8H.

This option works in conjunction with the Infared (IR) device option. If you can installing IR device, you needs to set this option as “Auto” for IRQ sharing for COM 2 device and IR device.

- Serial Port2 Mode

This option is not available if the setting of the Onboard Serial Port2 option is Disabled.  This option specifies the serial port 2 mode.  The settings are Standard, IrDA, or ASK IR (the ASK Computer infrared specification).  The Optimal and Fail-Safe default settings are Standard.

- IR Transmission Mode

This option specifies the type of duplex transmission used by serial port 2.  This option is only valid if the Serial Port2 Mode option is not set to Normal.  The settings are Half Duplex or Full Duplex. The Optimal and Fail-Safe default settings are N/A.

- Receiver Polarity

This option specifies the manner in which the infrared receive transmission signal is active.  The settings are Active High or Active Low.  The Optimal and Fail-Safe default settings are N/A.

- Transmitter Polarity

This option specifies the manner in which the infrared send transmission signal is active.  The settings are Active High or Active Low.  The Optimal and Fail-Safe default settings are N/A.

- Onboard Parallel Port

This option specifies the base I/O port address of the parallel port on the motherboard.  The settings are Auto (AMIBIOS automatically determines the correct base I/O port address), Disabled, 378h, 278h, or 3BCh.  The Optimal and Fail-Safe default settings are 378H.

- Parallel Port Mode

This option specifies the parallel port mode.  The Optimal and Fail-Safe default settings are Normal.  The settings are:

| Setting | Description |
| --- | --- |
| Normal | Specify this option to use the standard 8-bit IBM PC/AT-compatible Centronics parallel port. |
| EPP | The parallel port can be used with devices that adhere to the Enhanced Parallel Port (EPP) specification. EPP uses the existing parallel port signals to provide asymmetric bidirectional data transfer driven by the host device. |
| ECP | The parallel port can be used with devices that adhere to the Extended Capabilities Port (ECP) specification. ECP uses the DMA protocol to achieve data transfer rates up to 2.5 Megabits per second. ECP provides symmetric bidirectional communication. |

The Optimal and Fail-Safe default settings are Normal.

- EPP Version

This option is only valid if the Parallel Port Mode option is set to EPP.  This option specifies the version of the Enhanced Parallel Port specification that will be used by AMIBIOS.  The settings are 1.7 or 1.9.  The Optimal and Fail-Safe default settings are N/A.

- Parallel Port IRQ

This option is only valid if the Onboard Parallel Port option is not set to Disabled. This option sets the IRQ used by the parallel port.  The settings are 5, 7, or Auto (AMIBIOS automatically sets the correct parallel port IRQ).  The Optimal and Fail-Safe default settings are Auto.

- Parallel Port DMA Channel

This option is only available if the setting of the Parallel Port Mode option is ECP. This option sets the DMA channel used by the ECP-capable parallel port.  The settings are None, 0 (DMA Channel 0), 1 (DMA Channel 1), or 3 (DMA Channel 3).  The Optimal and Fail-Safe default settings are 3

- Onboard IDE

This option specifies the channel used by the IDE controller on the motherboard.  The settings are Disabled, Primary, Secondary, or Both.  The Optimal and Fail-Safe default settings are Primary.

For Fast Configuration

- Optimal Defaults (Default setting)

The Optimal default values provide optimum performance settings for all devices and system features.

- Fail-Safe Defaults

The Fail-Safe default settings consist of the safest set of parameters.  Use them if the system is behaving erratically.  They should always work but do not provide optimal system performance characteristics.

LIMITED WARRANTY

Congratulations on your purchase of an the Motherboard . This product is engineered to give many years of trouble free performance.  This limited warranty is provided for your protection.

The warrants this product to be in good working order for a period of one year from date of purchase as a new product.  In the event of failure of any part(s) due to defects in material or workmanship occurring within that one year period, DataExpert will, at its option, either repair or replace the defective product or part thereof at no charge for the parts or labor.

The limited warranty described above is in addition to whatever implied warranties may be granted to purchaser by law.  To the extent permitted by applicable law, ALL IMPLIED WARRANTIES INCLUDING THE WARRANTIES OF MERCHANTABILITY AND FITNESS FOR USE ARE LIMITED TO THE PERIOD FROM THE DATE OF PURCHASE FOR ONE YEAR.  Some states do not allow limitations on how long an implied warranty lasts, so the above limitations may not apply to you.

The limited warranty is non-transferable and does not apply if the product has been damaged by abuse, misuse, modification, accident, misapplication, or shipment to DataExpert.  Service by anyone other than an authorized DataExpert service agent will also render this limited warranty void. It is the purchaser's responsibility to determine the suitability of this product for your purposes.  DataExpert does not warrant that this product will meet your requirements.

LIABILITY DISCLAIMER

The product described in this manual is warranted in accordance with the terms of the applicable DataExpert product specifications.  Product performance is affected by system configuration, software, application, customer data, operator control, and other factors.

RMA FORM

When the motherboard can not work well, please fill up this form to describe related situations.  If the space is not enough to use, you can attach separate paper.

MODEL:				MODEL NO:

HARDWARE

CPU: Brand		, Model 	     		, Speed 			MHz

CD-PROCESSOR: Brand	, Model 		, Speed		MHz

SIMM: Brand                             , Speed                        ns, Q’ty                 pcs, Total            MB

CACHE: Brand                            , Speed                       ns, Total                K

TAG RAM: Brand                       , Speed                       ns

BIOS DATA CODE:

SYSTEM SPEED RUNNING                             MHz

VIDEO CARD:  Chip                             , RAM                            , VGA Mode

Bus                                        (ISA, VESA or PCI)

OTHER ADD-ON CARDS:

SOFTWARE

OPERATING SYSTEM                                                VERSION

SOFTWARE PROGRAM

BIOS SETUP: DRAM Wait State                               CACHE Wait State

If you change BIOS SETUP, please describe the changes:

<A>ERROR

¨HANG UP    	¨NO SCREEN   	¨FLOPPY R/W ERROR

¨HARD DISK R/W ERROR    	¨PARITY MEMORY ERROR

¨OTHER

<B>ERROR MESSAGES ON YOUR SCREEN (PLEASE SHOW US THE WHOLE SENTENCE)

<C>PROBLEM DESCRIPTION